Thermodynamics: Concepts and Applications

Chapter 2: Thermodynamic Properties, Property Relationships and Processes

2.2 – Frequently used Thermodynamic properties:

2.2c – Properties Related to the Second Law

- Entropy is a measure of the unavailability of thermal energy to do work in a closed system

- Entropy is a measure of the microscopic randomness associated with a closed system

- Gibbs Free Energy:

G≡H-TS

g≡h-Ts

- It is useful for degining equilibrium conditions for reacting systems at constant pressure and temperature.

2.4 – Ideal Gases as Pure Substances:

2.4g – Processes in T-s and P-v Space:

- Now we have four properties that define states and describe processes (P, v, T and s)

P2P1=v2v1-γ

T2T1=v2v11-γ

- Note that v here is specific volume, not velocity

- In T-s space:

  - Constant T is horizontal line, constant s is vertical line

  - Constant P is upward slope (you know this)

  - Constant v is same as constant P but steeper

- In P-v space:

  - Constant T is downward slope, opposite of constant p in the previous

  - Constants is steeper slope downwards.

Chapter 3: Conservation of Mass

m=ρvxA

- If the velocity is not constant

m=AρvxdA

Chapter 5: Conservation of Energy

5.2 – Energy Conservation for a System:

- Consider a system of fixed mass.

- If we look at the equations from Chapter 1 and apply them to energy:

Ein-Eout+Egenerated=∆Estored

- Similarly:

Ein-Eout+Egenerated=Estored

- Think about it, the energy in the system is equal to what comes in minus what goes out plus what is generated.

- So now, we try to associate each of these terms with particular forms of energy for various physical situations.

  - Our only restriction is the exclusion of nuclear transformations

5.2a – General Integral Forms:

- Consider a macroscopic system, several forms of enery can be associated with it.

  - The system always posseses internal energy

    - Thermal Energy

    - Chemical Energy

  - May also contain bulk kinetic and potential energies

Esys=Usys+(KE)sys+(PE)sys

- These energies are contained within the system boundary.

- However, the system may lose or gain energy by transfer across the system boundary.

  - These transfers occur only as a result of heat and/or work interactions with the surroundings

- System gains energy if heat is transferred into it or if work is done on it

- System loses energy if heat is transferred out of it or if it does work on surroundings

- Note that Egenerated here is zero.

- Consider incremental quantities of energy across the system boundary over time interval dt either

  - Into the system: δEin

  - Out of the system: δEout

- These result in an incremental change in the amount of energy stored within the system, dEsys

δEin-δEout=dEsys

- δEin is the work performed on the system and the heat added

- δEout is the work done by the system and the heat out

- So

δQin-δQout-δWout-δWin=dEsys

δQin, net-δWout, net=dEsys

- Note: “Q and W have no meaning inside the boundary” it is what happens to the boundary relative to surroundings

- If we look at changes of state and integrate these last two equations, we basically get:

1Qin2-1Win2-1Qout2-1Wout2=∆Esys

1Qin, net2-1Wout, net2=∆Esys

- The 1s and 2s are supposed to be subscript,

- This notation does not represent a change from state 1 to 2, it represents an integration of δQ and δW over particular paths!

- Q and W are path functions

- E is a state function

∆Esys≡Esys, 2-Esys, 1

- Note: E is the total energy of the system, for a stationary system (Like the gas in a piston) sometimes you can replace E2 – E1 with U2 – U1

- For an ideal gas, internal energy is a function of temperature only.

- You often assume it is a quasi-equilibrium process

At an Instant

- Basically looking at rate from energy conservation:

Ein≡Win+Qin

Eout≡Wout+Qout

Egenerated≡0

- and

Estored≡dEsysdt

- Q dot and W dot are the change in Q and W over change in t as t approaches 0

- Basically:

dEsysdt=Win+Qin-Wout+Qout

- Or, alternatively:

dEsysdt=Qin, net-Wout, net

5.2b – Reacting Systems:

- “Combustion is super duper important”

- Constant Pressure Combustion:

  - Consider a piston-cylinder system that contains a gaseous mixture of fuel and oxidizer at first, then combustion products at the end.

  - Same pressure

  - We assume kinetic and potential energy are 0

  - We get:

1Qin, net2-1Wout, net2=U2-U1

  - Subscripts 1 and 2 refer to unburned and burned states.

  - Assume no friction and quasi-static so pressure is uniform and constant:

1Qin, net2-PV2-V1=U2-U1

  - If you move the PC terms to the right hand side, you can again substitute enthalpy for U+PV. Then take out mass for intensive properties obviously

  - Also we switch the h1 and h2 terms around so that we look at Qout instead of in:

Qout=Mhreact-hprod

  - If you know the initial and final mixture compositions and temperatures, you can use tabulated values to find h

  - Note that for a constant-pressure combustion AND adiabatic process, there is a maximum temperature that can be reached: Tad

    - “Constant-pressure adiabatic flame temperature”

    - For such a process, the initial enthalpy equals the final enthalpy!

- Constant-Volume Combustion:

  - We start again with:

1Qin, net2-1Wout, net2=U2-U1

  - But since the change in volume is 0, the work term drops out

  - We also rewrite it as Qout, not Qin

Qout=Mureact-uprod

  - To evaluate the constant volume adiabatic flame temperature you set Qout = 0

  - Then you know that U is a function of T and P so for both sides U(T, P) but for the initial and final states

  - You get the exact same curve as in Figure 5.5 but instead of h, it’s u

  - Also, since most tables only have h instead of u, we rearrange the equation:

Hreac-Hprod-VPinit-Pf=0

  - Then you can substitute NRT for the PVs

  - You can also do some substitution with N=M/Mm to get:

hreac-hprod-RuTinitMm,reac-TadMm,prod=0

  - Mm is the molar mass

Skipped the two sample problems, might be worth going over them

5.3 – Energy Conservation for Control Volumes:

- We start by developing steady-state expressions of energy conservation for integral control volumes, followed by the unsteady case

- For steady-state and steady flow, properties do not change with time within the control volume or within the inlet and outlet streams.

min=mout

- We assume that fluid properties are uniform where the flow crosses the control surface

- We begin by identifying all the energy interactions around the control surface:

  - The inlet stream carries energy inside the control volume (mein) and the exit stream carries energy out of the control volume (meou)

  - Flow work is performed by the surroundings to push the fluid into the control volume and the control volume performs work to push the fluid out.

  - The control volume may also perform work that crosses the control surface at locations other than the fluid inlets and exit (Wcv, net out) FOR EXAMPLE: SHAFT WORK!

  - Finally, there will be heat transfer into the control volume if there are temperatyre gradients.

- THOSE ARE ALL THE ENERGY INTERACTIONS THE CONTROL SURFACE CAN HAVE.

- So we apply this conservation of energy principle:

  - “In steady state, the net rate at which energy crosses the control surface must be zero; that is, the rate at which energy enters the control volume must equal the rate at which energy exits.”:

ECS, in=ECS, out

- If we take all the energies we just described into account, that gives:

QCV, in+WCV, in+Wflow, in+mein=QCV, out+WCV, out+Wflow, out+meout

- Rearranging:

QCV, in-QCV, out+WCV, in-WCV, out=m(eout-ein)+Wflow, out-Wflow, in

- e is the specific energy (energy per unit mass) and it equal to the internal, kinetic and potential energies of the fluid

  - So you can rewrite it as e = u + (ke) + (pe)

- Also the flow work is equal to ṁPv

  - V is specific volume

  - So when you substitute that and factor m out, you can combine u and PV into h

- So you can substitute all those things in and then factor out the ṁ:

QCV, in-QCV, out+WCV, in-WCV, out=mhout-hin+12Vout2-Vin2+gzout-zin

- Which can more concisely be written as:

QCV, net in-WCV, net out=m∆h+∆ke+∆pe

Chapter 6: Second Law of Thermodynamics and Some of its Consequences

6.1 - Historical Context:

- Sadi Carnot worked to understand the relationship between heat supplied to an engine and the work it produced

- Reversible cycles are the most efficient cycles of all.

- The efficiency of the conversion of heat to work depends only on the temperatures at which an engine receives and rejects heat

- Rudolf Clausius defined the term entropy and is credited with the discoverer of the second law.

6.2 - Usefulness of the Second Law:

- It establishes theoretical limits of performance of cycles, engines and other energy conversion devices. It also provides means to quantitatively compare real devices with these theoretical ideals.

- Determines the direction for any spontaneous change and determines the equilibrium state of any system.

- Other things too like a temperature scale independent of the properties of any substance.

6.3 – One Fundamental Statement of the Second Law:

- Statements of the first law:

  - Though all work can be converted completely to heat, heat cannot be completely and continuously converted into work.

  - Kelvin-Planck Statement: It is impossible to construct a cyclically operating device for which the sole effect is exchange of heat with a single reservoir and the creation of an equivalent amount of work.

- So not all forms of energy are equally valuable

Skipped some stuff

6.3a – Reservoirs

- A heat reservoir is a source of heat energy sufficiently large that extracting any desired amount of energy won’t change it’s temperature.

  - E.g. earth’s atmosphere, oceans, lakes and rivers

  - You can take energy from those without changing their temperature (an AC does that)

6.3b – Heat Engines

- A heat engine is the cyclically operating device in the Kelvin-Planck statement.

There’s a good picture to add here

- The engine takes high energy, converts some to work and rejects the rest to a low-temperature reservoir.

  - The working fluid then returns to its original state with work input

- The first law relates the heat supplied and rejected during the cycle to the net work produced:

δQ-δW=dU

δQ-δW=∆U=0

- The internal energy change is 0 because the initial and final states are identical.

- Heat and work are path functions

QH-QL-Wnet=0

Wnet=QH-QL

- QH is the energy in and out of the high-temperature reservoir, QL is the energy in and out of the low-temperature reservoir

- In the example of the Rankine-cycle steam power plant, you don’t include the work in the water pump

- A heat pump removes energy from the atmosphere, ground or body of water and delivers energy to provide residential heating. The desired effect is to supply energy to the high-temperature reservoir.

  - The desired effect for a refrigerator is the removal of energy from the low – temperature reservoir.

Skipped some more stuff (a bit)

6.3c – Thermal Efficiency

- Thermal efficiency (first law efficiency) for heat engines is the ratio of net useful work produced to the heat energy supplied

ηt≡Useful Work producedEnergy supplied

ηt=WnetQH=QH-QLQL

ηt=1-QLQL

- For the efficiency of a heat pump, it’s the output heat over input energy.

- For the efficiency of a refrigerator, it’s the renergy removed from othe cold space over energy input.

6.3d – Reversibility

I skipped all of this except for:

- “A reversible process is one such that the system and all parts of the surroundings can be restored to their initial states”

6.4 – Consequences of the Kelvin-Planck Statement:

- Any and all heat engines must reject a portion of the heat energy suppled; thus, their thermal efficiency can never be 100%

  - You can’t continuously convert heat from a single reservoir to work

  - You need a second reservoir and the rejection of some heat

  - It is impossible to close the cycle without rejecting some heat.

- Efficiency of a reversible engine is solely determined by temperatures of hot and cold reservoir.

6.4b – The Carnot Efficiency:

ηt=1-TLTL

- For a heat pump:

βrev, heat pump=THTH-TL

- For a refrigerator:

βrev, heat pump=TLTH-TL

6.4c – Some reversible cycles:

- The Carnot cycle:

  - Reversible heat addition at constant temperature

  - Reversible adiabatic expansion

  - Reversible heat rejection at constant temperature

  - Reversible adiabatic compression

6.6 – Entropy Revisited

6.6e – Isentropic Efficiency:

- We use entropy to quantify performance of practical devies

ηisen,  t=WactWisen